|  | Risk Assessment | Page 1 of 2 Issue v3 Date form issued: 09/08/11 Revised: 09/08/11 Form No:HST/22 |
| --- | --- | --- |

| Task/Activity: Grinding Stone | Date assessment completed: 01/011/2015; Review Date: 01/11/2016. | Date assessment completed: 01/011/2015; Review Date: 01/11/2016. |
| --- | --- | --- |
| Brief Details of Task/Activity 1. 1. Clear appropriate working area to permit access and facilitate task – the general use of bonded grinding discs mounted machines.2. Ensure that machine is operating correctly and that guards are in place.3. Ensure that the wheel is dressed correctly and the tool rest is adjusted in accordance with the work to be undertaken i.e. as close to the wheel as possible.4. Clear work area upon completion of the work and remove any unused materials. | # Assessment completed by: LXC | Signature: |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cuts from use of hand tools/handling equipment/materials etc. | Moderate torn muscle | - Where appropriate use gloves when handling. - Sweep up debris where possible & avoid handling it. - Ensure tools are appropriate for the task, maintained & kept in good order. Use in the correct fashion for the task at hand. |  | Minor | LXC |  |  |
| Contact with wheel/disc in motion. | Very Severe e.g. death, permanent disability | - Only trained personnel to use this type of equipment. - Use tool on firm level base. - Keep hands as far away as possible from moving parts. - Ensure machine is isolated during tool changes and servicing note run down time as per working procedures. |  | Major |  |  |  |
| Ejection of disc/wheel | Very Severe e.g. death, permanent disability | - Only trained and authorised (in writing) staff to change abrasive wheels and discs. - Each appointee should carry proof of authorisation. - Correct type of wheels/disc to suit the type of work and spindle speeds, etc. should be available. |  | Catastrophic |  |  |  |
| Contact with machinery | Very Severe e.g. death, permanent disability | - Wherever practicable, abrasive wheels should be fitted with working guards designed to contain every part of the wheel if it fractures. - Good ventilation should be in place if dust is a hazard - Abrasive wheels should be stored in a dry area that is not susceptible to extreme temperatures. - Proper storage facilities for wheels and discs should be available. - All new discs should be inspected for damage before being fitted. - No loose clothing should be worn. |  | Major |  |  |  |
| Ejection of materials | Very Severe e.g. death, permanent disability | - Wear eye protection i.e. to protect against dust/penetration injury - Avoid contact with sealant/adhesive/lubricants. |  | Major |  |  |  |
| Slips and Trips | Very Severe e.g. death, permanent disability | - Ensure that good housekeeping at workplace is maintained to acceptable health & safety standards - Ensure lighting at place of work is adequate. Avoid stroboscopic affect. Use/provide additional task lighting where required. - Ensure that materials are stored within the allocated/demarcated areas to avoid encroaching onto walkways etc. - Ensure that any dustsheets etc. used are placed and fixed so as not to produce a tripping/slipping hazard. |  | Major |  |  |  |
| Hazardous substances | Very Severe e.g. death, permanent disability | - Avoid contact with sealant/adhesive/lubricants/dust. - Wear gloves when handling irritants. - Good hygiene principals i.e. wash hands before eating, drinking and after work is completed. - Report any persistent irritation to your supervisor/Occupational. Health - Ensure adequate ventilation/Use LEV as required. Ensure that the LEV is examined and tested on a 12 monthly basis by a competent person and written record of the examination is kept. - Ensure adequate ventilation to working area by means of opening doors and windows etc |  | Significant |  |  |  |
| Entanglement with machinery | Very Severe e.g. death, permanent disability | - Ensure all guards are in place. Paying particular attention to any unguarded spindles i.e. protective sheath locked in place - Wear appropriate clothing, tie back long hair and remove hanging jewellery - Readily accessible stop controls that are easy to use and can be activated from the operators position. If not then additional emergency stop buttons must be fitted. These will need to be made known to all machine operators. - Emergency stop buttons must be located around the workshop and be clearly marked with the appropriate sign. They must be set to “0 volts” bringing all workshop machinery to a halt. - Emergency stops should be checked on a regular basis and kept accessible at all times. - Loose clothing must be removed or securely fastened back. Jewellery, which could become entangled, must be removed. - Students must not use equipment unless supervised and must be instructed in the safe use of the machine with regular reminders to reinforce this point. - Fixed equipment should be regularly inspected by Technology Department Staff for obvious defects. - Any fixed equipment found to be faulty must be removed from service by isolating the power supply, so that it cannot be inadvertently switched on, and attaching a notice. |  | Minor |  |  |  |
| Fire | Very Severe e.g. death, permanent disability | - Ensure good housekeeping at site and do not smoke. - Ensure that waste material is cleaned away and items are placed in the metal bins prior to re use or disposal. |  | Minor |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk, requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk, requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk, requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |