|  | Risk Assessment | Page 1 of 2 Issue v3 Date form issued: 09/08/11 Revised: 09/08/11 Form No:HST/22 |
| --- | --- | --- |

| Task/Activity: Orbital Sander / Belt Sander | Date assessment completed: 01/011/2015; Review Date: 01/11/2016. | Date assessment completed: 01/011/2015; Review Date: 01/11/2016. |
| --- | --- | --- |
| Brief Details of Task/Activity 1. Obtain the material to be sanded. 2. Place the material on to a suitable work surface.3. Clamp the material to the work surface.4. Collect the power tool from the stores.5. Visually inspect the power tool working parts, housing, plug and lead for defects (if defects are noted the equipment is taken out of use and repaired).6. Connect power tool to power supply and switch on supply.7. Carry out the task.8. Switch off electrical supply and disconnect the power tool.9. Clean the power tool of debris with a soft brush.Replace back into the store. | # Assessment completed by: LXC | Signature: |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contact with moving parts | Very Severe e.g. death, permanent disability | - Moving parts guard to be in place. - Hands to be away from the moving parts. |  | Significant |  |  |  |
| Entanglement with machinery | Very Severe e.g. death, permanent disability | - Suitable overalls to be worn. - Long hair should be prevented from contact with moving parts via tying up. |  | Significant |  |  |  |
| Dust and ejection of materials | Very Severe e.g. death, permanent disability | - Extraction equipment to be used where practicable. - Operative to wear eye protection, facemask, and overalls. - A suitable restricted area for students around the operation should be maintained. - Students should not be allowed in the area when the task is in operation. |  | Significant |  |  |  |
| Slips and trips | Very Severe e.g. death, permanent disability | - A safe system of working should be implemented. - Ensure good housekeeping - Ensure that the area is clear and free from debris. |  | Minor |  |  |  |
| Noise |  | - A program of noise monitoring should take place and the recommendations implemented. The survey must be carried out by a competent person |  | Minor |  |  |  |
| Vibration |  | - A safe system of working should be implemented. No prolonged process to be carried out without regular rest breaks. |  | Major |  |  |  |
| Electricity | Very Severe e.g. death, permanent disability | - A program of portable electrical appliance testing is to be carried out on a regular basis by a competent person. - The operative before each use should visually inspect the equipment. - The equipment should be powered via a residual current device (RCD) |  | Major |  |  |  |
| Fire | Very Severe e.g. death, permanent disability | - Ensure good housekeeping at site and do not smoke. - Ensure that waste material is cleaned away and items are placed in the metal bins prior to re use or disposal. |  | Significant |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk, requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk, requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk, requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |