|  | Risk Assessment | Page 1 of 2 Issue v3 Date form issued: 09/08/11 Revised: 09/08/11 Form No:HST/22 |
| --- | --- | --- |

| Task/Activity: Work Shop | Date assessment completed: 01/011/2015 | Review Date: 01/11/2016 |
| --- | --- | --- |
| Brief Details of Task/Activity | # Assessment completed by: LXC | Signature: |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inadequate assessment of risk | Staff / Pupils | - CLEAPSS Risk Assessments in Technology / DATA risk assessment and BS 4163:2007 followed in secondary schools and adapted in light of local conditions - Incorporated into materials normally used in teaching – schemes of work, lesson plan, worksheets etc. recording outcomes |  |  |  |  |  |
| Supervision / class sizes | Staff / Pupils | - Group size appropriate to the design and size of the room, take account of the nature of the task, the equipment, the age, ability, aptitude and special education needs of pupils. - Maximum of 20 pupils with a competent /qualified teacher - Adequate supervision in place. - Rules displayed in workrooms |  |  |  |  |  |
| Competency of staff and pupils | Staff / Pupils | - Staff trained in the safe use of equipment, machinery and materials and have undertaken DATA specialist extension modules - DATA training reaccreditation every 5 yrs - Health and safety practice forms part of all curriculum work. - Safety instruction given before machine is operated and is this recorded for pupils |  |  |  |  |  |
| Defective flooring Spillages Items stored in walkways Slips Trips Falls | Staff / Pupils | - Condition of rooms regularly checked. Prompt maintenance of defects. - Good housekeeping maintained - Designated storage areas .Wood and metal store tidy, adequate racking and away from teaching areas. - Floor surfaces unobstructed and slip free - Immediate cleaning up of spillages - Doors and gangways unobstructed - No accumulation of rubbish / combustible waste. |  |  |  |  |  |
| Unauthorised use of machinery / workshop | Staff / Pupils | - Workshop locked when unoccupied - Power is locked off when workshop is unsupervised - Equipment which must not be used by pupils (circular saws, planers, bandsaw etc) kept strictly out of bounds to pupils / fitted with key operated isolating switch and locked off when not in use. |  |  |  |  |  |
| Use of Equipment / tools | Staff / Pupils | - CLEAPSS Risk Assessments in Technology / DATA risk assessment and BS 4163:2007 followed in secondary schools and adapted in light of local conditions. - Health and safety issues covered as part of curriculum - Equipment guards are always in use and properly adjusted - Adequate space around machinery (marked on floor 600mm with yellow line) - Relevant safety signage on specific machinery. - In house equipment checks at least annually ( termly recommended in HCC DT handbook) - Machinery inspected and serviced annually by a competent engineer - Breaks fitted to relevant machines ( circular saw, bandsaw, planer thicknesser) to stop blade and motor in 10s. - Wall mounted emergency stops in place within workshop which disconnect and isolate all electrical circuits other then lighting - Approved protective clothing and eye protection worn as recommended - Eye protection stored adjacent to relevant machinery |  |  |  |  |  |
| Sharp blades / Cuts | Staff / Pupils | - Tools allocated under supervision checked in after use and stored tidily in racks - Visual inspection pre-use |  |  |  |  |  |
| Storage | Staff / Pupils | - Equipment and substances stored appropriately. - Heavy items stored at the appropriate level. - Flammable liquids (kept to a minimum and must not exceed 50 litres). - All highly flammable substances should be stored in suitably labelled, lockable metal storage bin or cupboard designed for the purpose. - Rags, waste or similar materials should not be allowed to accumulate or be stored near naked flames, very hot objects or immediately against electrical equipment.. |  |  |  |  |  |
| Substances including Paints, inks, varnishes etc / may be toxic, cause allergic reactions, be irritants or react with other substances | Staff / Pupils | - Inventory of chemicals used within the department in place, identifies the quantity stored, that material safety data sheets have been obtained, level of risk from the substance has been identified - Any known hazards from substances and appropriate controls must be communicated to people who may be exposed to any risk, including teaching, cleaning staff and pupils. - Are hazardous materials stored appropriately (locked and signed) - Bottles >1 L stored at low level. Individual drip trays in place / drip tray +10% of largest container - < 50L of highly flammables stored within workroom |  |  |  |  |  |
| Inhalation of dusts | Staff / Pupils | - Wood working machines (circular saws, planer thicknessers, band saws etc.) which generate substantial quantities of dust fitted with extraction equipment interlocked with the machine - Extraction systems (LEV) checked and serviced regularly (14 months max) - Cleaning of filters on LEV undertaken weekly – appropriate PPE (FPP2 mask) - Regular cleaning regime to keep dust to a minimum. - Floors washed or damp-cleaned and vacuumed. HEPA vacuum available in woodwork areas - Appropriate cleaning methods communicated to cleaning staff . | All Staff to be trained on the machines Dust Extractors to be serviced. completed September 2012 |  |  |  |  |
| Electricity / Electric shock | Staff / Pupils | - Electrical equipment subject to regular safety inspection and test ('PAT testing') - Mains powered portable equipment protected by RCD to distribution board, wall socket or lead (RCD’s checked monthly and log kept) - Isolators for all circuits in readily accessible positions and local isolators fitted to each fixed machine. - Sufficient outlets to support the range of equipment normally used with resorting to regular use of multi-socket adaptors. |  |  |  |  |  |
| Gas supplies and gas burning appliances | Staff / Pupils | - All mains gas supplies to equipment controlled in each work area by a clearly marked emergency master valve. - Gas supply isolated when not in use - Fixed supply tested annually |  |  |  |  |  |
| First Aid | Staff / Pupils | - At least one first aid kit in the department of a secondary school. - DT staff have basic knowledge of first aid. - Contents regularly checked. - Suitable eyewash facilities available (eyewash bottles min 300ml total of 900ml or sterile tube / mains water supply) |  |  |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk, requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk, requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk, requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |